Course 2: Control A PX4 Drone With An LLM

From a completely fresh Ubuntu 24.04 installation to a full ROS 2 + PX4 + Gazebo + Ollama + YOLO workflow.

These notes assume a completely fresh Ubuntu 24.04 Desktop machine. Nothing is assumed to be preinstalled except Ubuntu itself and an internet connection.

Goal: by the end of this handout, students will be able to launch PX4 SITL in Gazebo, bridge the camera into ROS 2, run the LLM-controlled drone stack, visualize YOLO detections, and command the drone with natural-language prompts.

# 1. Fresh Ubuntu Preparation

Open a terminal and run:

sudo apt update sudo apt upgrade -y sudo apt install -y \ software-properties-common \ curl \ git \ wget \ gnupg \ lsb-release \ locales \ build-essential \ cmake \ python3-pip \ python3-colcon-common-extensions

Set a UTF-8 locale:

sudo locale-gen en_US en_US.UTF-8 sudo update-locale LC_ALL=en_US.UTF-8 LANG=en_US.UTF-8 export LANG=en_US.UTF-8 locale

# 2. Install ROS 2 Jazzy

Enable the Ubuntu universe repository:

sudo apt install -y software-properties-common sudo add-apt-repository universe -y

Add the official ROS 2 apt source:

sudo apt update && sudo apt install curl -y export ROS_APT_SOURCE_VERSION=$(curl -s https://api.github.com/repos/ros-infrastructure/ros-apt-source/releases/latest ¦ grep -F '"tag_name"' ¦ awk -F'"' '{print $4}') curl -L -o /tmp/ros2-apt-source.deb "https://github.com/ros-infrastructure/ros-apt-source/releases/download/${ROS_APT_SOURCE_VERSION}/ros2-apt-source_${ROS_APT_SOURCE_VERSION}.$(. /etc/os-release && echo ${UBUNTU_CODENAME:-${VERSION_CODENAME}})_all.deb" sudo dpkg -i /tmp/ros2-apt-source.deb

Install ROS 2 Jazzy Desktop and development tools:

sudo apt update sudo apt install -y ros-jazzy-desktop ros-dev-tools

Add ROS 2 to your shell and verify it:

echo "source /opt/ros/jazzy/setup.bash" >> ~/.bashrc source /opt/ros/jazzy/setup.bash ros2 --version

# 3. Install Gazebo ROS Integration And Tools

sudo apt update sudo apt install -y \ ros-jazzy-ros-gz \ ros-jazzy-ros-gz-bridge \ ros-jazzy-ros-gz-image \ ros-jazzy-cv-bridge \ ros-jazzy-rqt-image-view

Optional but useful shell settings:

echo 'export PATH=/usr/bin:$PATH' >> ~/.bashrc echo 'export GZ_CONFIG_PATH=/usr/share/gz' >> ~/.bashrc source ~/.bashrc

# 4. Install PX4-Autopilot

Clone PX4:

cd ~ git clone --recursive https://github.com/PX4/PX4-Autopilot.git

Run the PX4 Ubuntu setup script:

cd ~/PX4-Autopilot bash ./Tools/setup/ubuntu.sh

When the script finishes, reboot the machine:

sudo reboot

# 5. After Reboot: Re-Source ROS 2

source /opt/ros/jazzy/setup.bash

# 6. Test PX4 + Gazebo

Launch the camera-equipped PX4 SITL model:

cd ~/PX4-Autopilot make px4_sitl gz_x500_mono_cam

Inside the PX4 shell, apply these parameters once:

param set NAV_DLL_ACT 0 param set COM_RC_IN_MODE 1 param set COM_ARM_SDCARD 0 param set CBRK_SUPPLY_CHK 894281 param save

Optional quick test in the PX4 shell:

commander arm commander takeoff

Stop PX4 with `Ctrl+C` before continuing.

# 7. Optional: Install QGroundControl

This is optional for the LLM course, but useful for monitoring vehicle state.

Prepare the system:

sudo usermod -aG dialout "$(id -un)" sudo systemctl mask --now ModemManager.service sudo apt install -y \ gstreamer1.0-plugins-bad \ gstreamer1.0-libav \ gstreamer1.0-gl \ python3-gi \ python3-gst-1.0 \ libfuse2 \ libxcb-xinerama0 \ libxkbcommon-x11-0 \ libxcb-cursor-dev

Now log out and log back in once so the `dialout` group takes effect.

Download `QGroundControl-x86_64.AppImage` from the official QGroundControl site into `~/Downloads`, then run:

cd ~/Downloads chmod +x QGroundControl-*.AppImage ./QGroundControl-*.AppImage

# 8. Install The PX4 DDS Bridge Agent

Build `MicroXRCEAgent` from source:

cd ~ git clone -b v2.4.3 https://github.com/eProsima/Micro-XRCE-DDS-Agent.git cd ~/Micro-XRCE-DDS-Agent mkdir build cd build cmake .. make -j$(nproc) sudo make install sudo ldconfig /usr/local/lib/

Verify it exists:

MicroXRCEAgent --help

# 9. Install Ollama

curl -fsSL https://ollama.com/install.sh ¦ sh

Start Ollama in a dedicated terminal:

ollama serve

Leave that terminal open. In a second terminal, pull the models used by this project:

ollama pull qwen3:4b ollama pull functiongemma ollama list

# 10. Clone The LLM Drone Project

cd ~ git clone https://github.com/mouad-boumediene/LLM-controlled-drone.git cd ~/LLM-controlled-drone

Replace the embedded `px4_msgs` placeholder with a real clone before building:

rm -rf ~/LLM-controlled-drone/src/px4_msgs git clone https://github.com/PX4/px4_msgs.git ~/LLM-controlled-drone/src/px4_msgs

# 11. Install Python Packages Used By The Project

Ubuntu 24.04 may block plain `pip install` into the system Python, so use:

/usr/bin/python3 -m pip install --break-system-packages "numpy<2" ultralytics opencv-python

# 12. Build The Workspace

Initialize `rosdep` once on a fresh machine:

sudo rosdep init

Then build the workspace:

source /opt/ros/jazzy/setup.bash cd ~/LLM-controlled-drone rosdep update rosdep install --from-paths src --ignore-src -r -y colcon build --symlink-install --packages-select drone_agent source ~/LLM-controlled-drone/install/setup.bash

Add the workspace to your shell:

echo "source ~/LLM-controlled-drone/install/setup.bash" >> ~/.bashrc source ~/.bashrc

# 13. Launch The Full System

If any terminal shows `(base)`, deactivate Conda first:

conda deactivate conda deactivate

## Terminal 1: Ollama

ollama serve

If it says Ollama is already running, that is fine.

## Terminal 2: PX4 + Gazebo

cd ~/PX4-Autopilot make px4_sitl gz_x500_mono_cam

## Terminal 3: Micro XRCE DDS Agent

MicroXRCEAgent udp4 -p 8888

## Terminal 4: Gazebo Camera Bridge

source /opt/ros/jazzy/setup.bash ros2 run ros_gz_bridge parameter_bridge /world/default/model/x500_mono_cam_0/link/camera_link/sensor/camera/image@sensor_msgs/msg/Image[gz.msgs.Image --ros-args -r /world/default/model/x500_mono_cam_0/link/camera_link/sensor/camera/image:=/x500/camera/image_raw

## Terminal 5: LLM Drone Stack

source /opt/ros/jazzy/setup.bash source ~/LLM-controlled-drone/install/setup.bash ros2 launch drone_agent drone_agent.launch.py camera_topic:=/x500/camera/image_raw

## Terminal 6: Prompt Terminal

bash ~/LLM-controlled-drone/run_prompt_chat.sh

## Terminal 7: See The Camera / YOLO Output

source /opt/ros/jazzy/setup.bash ros2 run rqt_image_view rqt_image_view

In `rqt_image_view`, select `/x500/camera/image_raw` for the raw camera or `/yolo/image_annotated` for the YOLO overlay.

Optional detection text terminal:

source /opt/ros/jazzy/setup.bash ros2 topic echo /yolo/detections

## Terminal 8: RViz Trail

source /opt/ros/jazzy/setup.bash source ~/LLM-controlled-drone/install/setup.bash rviz2

In RViz:

1. Set `Fixed Frame` to `map`

2. Add `Path` and choose `/drone/path`

3. Add `Pose` and choose `/drone/pose`

# 14. Quick Health Checks

Check the ROS nodes:

source /opt/ros/jazzy/setup.bash ros2 node list

You should see at least `/brain_node`, `/yolo_detector`, and `/trail_node`.

Check the main LLM model:

source /opt/ros/jazzy/setup.bash ros2 param get /brain_node ollama_model

Expected:

String value is: qwen3:4b

Check GPS:

source /opt/ros/jazzy/setup.bash ros2 topic echo --once /fmu/out/vehicle_gps_position

If `fix_type` is `3` or higher, GPS-aware prompts should work.

# 15. First Prompt Sequence

reset memory

take off and hover at 5 meters

fly a heart shape

hold

fly a clover shape

hold

search for a person and stop above them when found

hold

search for a bus then approach it and hold for 10 seconds then search for a person and approach it

land

# 16. Good Prompt Examples For The Course

## Basic control

take off and hover at 3 meters go 5 meters north set altitude to 10 meters face east set speed to 1 meter per second hold land return to launch

## Shapes

fly in a circle of 5 meter radius fly a square pattern fly an upward spiral fly a figure eight fly a heart shape fly a clover shape

## Search and follow

search for a person approach the person move slowly toward the person follow the person search for a bus approach the bus

## Multi-step missions

takeoff, then set the altitude to 2 meters, then fly in a square for 1 minute, then hold for 10 seconds, then fly in a circle for 1 minute

search for a bus then approach it and hold for 10 seconds then search for a person and approach it

# 17. Rules To Tell Students

Run only one `drone_agent.launch.py` at a time

Run only one camera bridge at a time

Run only one `MicroXRCEAgent` at a time

If the drone is doing the wrong thing, type `hold`

A new prompt interrupts the current mission

If YOLO is not seeing the target, search/follow prompts will not behave well

Keep Ollama running the whole time

# 18. Full Cleanup Command

If the system gets messy and you want to kill everything:

pkill -f 'ros2 launch drone_agent drone_agent.launch.py' ¦¦ true pkill -f '/install/drone_agent/lib/drone_agent/brain_node' ¦¦ true pkill -f '/install/drone_agent/lib/drone_agent/yolo_detector' ¦¦ true pkill -f '/install/drone_agent/lib/drone_agent/trail_node' ¦¦ true pkill -f 'ros2 run ros_gz_bridge parameter_bridge /world/default/model/x500_mono_cam_0/link/camera_link/sensor/camera/image' ¦¦ true pkill -f 'MicroXRCEAgent udp4 -p 8888' ¦¦ true pkill -f '/PX4-Autopilot/build/px4_sitl_default/bin/px4' ¦¦ true pkill -f 'gz sim' ¦¦ true pkill -f 'ollama serve' ¦¦ true

Then launch again from Section 13.

# Official References

ROS 2 Jazzy Ubuntu install: https://docs.ros.org/en/jazzy/Installation/Alternatives/Ubuntu-Install-Binary.html

PX4 Ubuntu development environment: https://docs.px4.io/main/en/dev_setup/dev_env_linux_ubuntu

PX4 uXRCE-DDS bridge: https://docs.px4.io/main/en/middleware/uxrce_dds

Ollama Linux install: https://docs.ollama.com/linux

QGroundControl Ubuntu install: https://docs.qgroundcontrol.com/master/en/qgc-user-guide/getting_started/download_and_install.html